CH-47F AFCS Evaluation

Stabilized at 90 Knots, Directional Pedal Separation:

In Trim.

AFCS Both.

Select 1, no more than ½ inch.

AFCS Both.

Select 2, same.

Pitch Axis Test:

AFCS 1 or 2, as required.

90 knots, in trim.

FOR Cyclic to 100 knots.

Release, return to 90 ± 5.

AFT Cyclic to 80 knots.

Release, return to 90 ± 5.

BEEP FOR, do not exceed 100 knots.

BEEP AFT.

Roll Axis and Coordinated Turn:

90 knots, in trim.

Level Roll/Heading ± 3º, ½ BALL.

Right 20º, hold with 5º, ½ Ball.

Level Roll/Heading ± 3º, ½ BALL.

Repeat LEFT.

BEEP Right 20, hold ± 5º

BEEP Left, same.

BEEP Level.

Repeat on other system.

DASH Actuator Low Rate Operation Check:

100 knots, in trim.

AFCS OFF, slow to 70.

AFCS ON, check AFCS and DASH Fail caution out in 20 sec.

AFCS OFF, accel to 100.

AFCS ON, check AFCS and DASH Fail caution out in 20 sec.