CH-47F AFCS Function Checks

Engagement Error:

Stabilized Hover approx 15 - 20’.

AFCS Switch 1 to OFF to 1.

AFCS Switch 2 to OFF to 2.

AFCS – BOTH.

Pitch Axis Test:

AFCS 1 or 2 as required.

Nose Up 3°, return within 2°, 1 ½ oscillations.

Nose Down 3°, same.

Pitch Trim Check:

Beep FOR

Beep AFT

Check other CYCLIC.

Roll Axis Attitude Hold Test:

Level ± 1°.

Beep Left then Right.

Check other CYCLIC.

Yaw Axis Test:

Right Pedal ½ inch, 30 degrees.

Capture heading below 1°/sec, ± 3°.

Repeat Left.

Swivels unlocked.

Repeat Right/Left, no capture.

Heading Hold/Roll Trim Check:

Level ± 1°.

Right 5 – 7°, heading/bank angle ± 3°.

Repeat Left.

REPEAT ALL CHECKS ABOVE ON OTHER SYSTEM.

WHEN FINISHED, AFCS – BOTH.