CH-47F Vertical Velocity Checklist

RADAR ALTIMETER CHECKS:

Stabilized Hover approx 20’.

Engage RAD Hold (Check VSD/HSDH displays RAD).

Thrust down 5’, return within 3’.

Beep Up 6X, increase 3 to 9’.

Beep DN 5 Sec, ROD 100 to 300 FPM.

RAD Alt Disengaged.

TRC/PH CHECKS:

AFCS Both.

MFD select HSDH.

Trim for Stabilized Hover.

Engage PH, note SOLID BALL.

FOR Cyclic: 3 - 6 Kts GS, note SHADED BALL.

FOR Cyclic: 10 - 15 Kts, note OPEN BALL, TRC Disengaged.

DECEL to Hover, note TRC/PH engaged.

REPEAT Rear, Left, Right.

Stabilized HOVER, FE at Door.

Beep FOR 6 times, note forward movement 3 – 9’.

Beep AFT 6 seconds, note aft movement 3 – 9’.

AFCS TRC/PH – OFF.